Supplementary Material

Risk-Calibrated Sensor Fusion for Underground Utility Mapping: A Simulation-Based Decision-Support Framework for Excavation Safety

Software https://doi.org/10.5281/zenodo.23307525; preprint https://doi.org/10.5281/zenodo.23307526. All values are simulation results under stated assumptions; intervals are 95% cluster-bootstrap percentile intervals over corridors (stratified by soil-by-utility cell) unless stated.

# S1. Scenario tables

Percentages. Sensitivity is among decided segments; false clearance is the share of released (called absent) segments that are truly occupied; unverified and released are shares of all segments; strikes per km are released occupied segments per km of corridor. ECE is the expected calibration error of the probability (mean over blocks, 95% interval over blocks). Compute is seconds per block of 3,500 corridors (100 segments each).

### baseline  (defaults; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.4 (24.7 to 26.1) | 59.3 (58.7 to 60.0) | 1.0 (1.0 to 1.0) | 0.0 (0.0 to 0.0) | 82.9 (82.5 to 83.4) | 98.3 (97.1 to 99.7) | n/a | 0.00 |
| GPR only (dual band) | 83.9 (83.6 to 84.2) | 23.9 (23.4 to 24.4) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 44.3 (43.8 to 44.8) | 21.2 (20.8 to 21.6) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 84.4 (84.1 to 84.7) | 23.4 (22.9 to 23.9) | 1.1 (1.1 to 1.1) | 0.0 (0.0 to 0.0) | 44.0 (43.5 to 44.6) | 20.6 (20.2 to 21.0) | 0.036 | 0.01 |
| v1.0 continuity filter, one pass | 97.7 (97.4 to 97.8) | 4.3 (4.0 to 4.7) | 0.3 (0.2 to 0.3) | 16.0 (16.0 to 16.0) | 29.8 (29.4 to 30.3) | 2.6 (2.4 to 2.8) | 0.030 | 0.02 |
| v1.0 filter, three repeat passes | 98.2 (98.0 to 98.4) | 3.4 (3.1 to 3.8) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 29.6 (29.1 to 30.2) | 2.0 (1.8 to 2.3) | 0.039 | 0.03 |
| Proposed, one pass | 98.4 (98.2 to 98.6) | 3.0 (2.7 to 3.4) | 1.7 (1.6 to 1.9) | 16.0 (15.8 to 16.3) | 28.9 (28.4 to 29.4) | 1.7 (1.6 to 1.9) | 0.044 | 0.83 |
| Proposed, three parallel passes | 99.3 (99.3 to 99.4) | 1.3 (1.1 to 1.5) | 1.8 (1.6 to 1.9) | 15.7 (15.5 to 16.0) | 28.4 (27.8 to 28.9) | 0.7 (0.6 to 0.8) | 0.026 | 1.76 |

Zone shares (A/B/C): 0.72 / 0.08 / 0.20.

### curved  (geom=curved; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.5 (24.8 to 26.2) | 48.2 (47.7 to 48.7) | 7.0 (6.6 to 7.3) | 0.0 (0.0 to 0.0) | 83.1 (82.6 to 83.5) | 80.0 (79.1 to 81.0) | n/a | 0.00 |
| GPR only (dual band) | 82.2 (81.9 to 82.6) | 18.7 (18.3 to 19.0) | 10.4 (10.1 to 10.7) | 0.0 (0.0 to 0.0) | 51.0 (50.6 to 51.6) | 19.1 (18.7 to 19.4) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 82.8 (82.5 to 83.1) | 18.3 (17.9 to 18.7) | 10.9 (10.6 to 11.3) | 0.0 (0.0 to 0.0) | 50.4 (50.0 to 50.9) | 18.5 (18.1 to 18.8) | 0.064 | 0.01 |
| v1.0 continuity filter, one pass | 96.3 (96.0 to 96.6) | 4.6 (4.3 to 4.9) | 11.8 (11.4 to 12.2) | 16.0 (16.0 to 16.0) | 36.0 (35.6 to 36.5) | 3.3 (3.1 to 3.6) | 0.076 | 0.02 |
| v1.0 filter, three repeat passes | 96.9 (96.6 to 97.1) | 3.9 (3.6 to 4.2) | 11.4 (11.0 to 11.8) | 16.0 (16.0 to 16.0) | 35.9 (35.4 to 36.4) | 2.8 (2.6 to 3.0) | 0.081 | 0.03 |
| Proposed, one pass | 97.3 (97.1 to 97.5) | 3.4 (3.2 to 3.7) | 10.0 (9.7 to 10.3) | 17.5 (17.3 to 17.7) | 35.4 (34.8 to 35.8) | 2.4 (2.3 to 2.6) | 0.046 | 0.81 |
| Proposed, three parallel passes | 98.1 (98.0 to 98.3) | 2.2 (2.1 to 2.4) | 3.7 (3.5 to 3.9) | 17.0 (16.7 to 17.2) | 37.6 (37.1 to 38.1) | 1.7 (1.6 to 1.8) | 0.023 | 1.78 |

Zone shares (A/B/C): 0.71 / 0.09 / 0.20.

### shifted  (geom=shifted; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.7 (24.9 to 26.6) | 30.8 (30.4 to 31.3) | 5.8 (5.6 to 6.2) | 0.0 (0.0 to 0.0) | 87.0 (86.6 to 87.4) | 53.7 (52.8 to 54.5) | n/a | 0.00 |
| GPR only (dual band) | 83.1 (82.7 to 83.5) | 9.2 (9.0 to 9.4) | 6.0 (5.7 to 6.2) | 0.0 (0.0 to 0.0) | 66.2 (65.7 to 66.6) | 12.2 (11.9 to 12.5) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 83.6 (83.2 to 84.0) | 9.0 (8.7 to 9.2) | 6.4 (6.2 to 6.7) | 0.0 (0.0 to 0.0) | 65.7 (65.3 to 66.1) | 11.8 (11.5 to 12.1) | 0.065 | 0.01 |
| v1.0 continuity filter, one pass | 96.9 (96.6 to 97.2) | 1.8 (1.7 to 2.0) | 6.2 (6.0 to 6.5) | 16.0 (16.0 to 16.0) | 51.2 (50.9 to 51.7) | 1.9 (1.7 to 2.1) | 0.072 | 0.02 |
| v1.0 filter, three repeat passes | 97.5 (97.3 to 97.8) | 1.5 (1.3 to 1.6) | 5.8 (5.5 to 6.1) | 16.0 (16.0 to 16.0) | 51.3 (50.9 to 51.7) | 1.5 (1.3 to 1.7) | 0.064 | 0.04 |
| Proposed, one pass | 97.7 (97.5 to 97.9) | 1.4 (1.2 to 1.5) | 5.7 (5.4 to 6.0) | 17.1 (16.9 to 17.4) | 50.5 (50.1 to 50.9) | 1.4 (1.2 to 1.5) | 0.050 | 0.83 |
| Proposed, three parallel passes | 98.4 (98.2 to 98.6) | 0.9 (0.8 to 1.0) | 1.8 (1.7 to 1.9) | 16.6 (16.3 to 16.8) | 52.6 (52.2 to 53.1) | 1.0 (0.8 to 1.1) | 0.034 | 1.86 |

Zone shares (A/B/C): 0.72 / 0.08 / 0.20.

### shifted frequent  (geom=shifted_frequent; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.9 (25.0 to 26.7) | 19.1 (18.8 to 19.4) | 5.8 (5.6 to 6.1) | 0.0 (0.0 to 0.0) | 89.5 (89.2 to 89.8) | 34.3 (33.7 to 34.8) | n/a | 0.00 |
| GPR only (dual band) | 82.3 (81.8 to 82.7) | 5.4 (5.3 to 5.5) | 6.2 (6.0 to 6.4) | 0.0 (0.0 to 0.0) | 76.2 (75.9 to 76.5) | 8.2 (8.0 to 8.4) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 82.8 (82.4 to 83.2) | 5.3 (5.1 to 5.4) | 6.6 (6.4 to 6.7) | 0.0 (0.0 to 0.0) | 75.8 (75.5 to 76.1) | 8.0 (7.8 to 8.2) | 0.089 | 0.01 |
| v1.0 continuity filter, one pass | 96.0 (95.6 to 96.2) | 1.3 (1.2 to 1.4) | 6.5 (6.2 to 6.7) | 16.0 (16.0 to 16.0) | 61.2 (60.9 to 61.5) | 1.6 (1.5 to 1.7) | 0.093 | 0.02 |
| v1.0 filter, three repeat passes | 96.8 (96.5 to 97.1) | 1.0 (0.9 to 1.1) | 6.0 (5.8 to 6.2) | 16.0 (16.0 to 16.0) | 61.4 (61.1 to 61.7) | 1.2 (1.1 to 1.4) | 0.080 | 0.03 |
| Proposed, one pass | 96.7 (96.5 to 97.0) | 1.0 (0.9 to 1.1) | 5.9 (5.7 to 6.1) | 18.0 (17.7 to 18.2) | 60.3 (60.0 to 60.7) | 1.2 (1.1 to 1.3) | 0.065 | 0.86 |
| Proposed, three parallel passes | 97.8 (97.6 to 97.9) | 0.7 (0.6 to 0.7) | 2.0 (1.9 to 2.1) | 17.1 (16.8 to 17.3) | 63.0 (62.7 to 63.4) | 0.9 (0.8 to 0.9) | 0.051 | 1.83 |

Zone shares (A/B/C): 0.72 / 0.08 / 0.20.

### interrupted  (geom=interrupted; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.7 (24.9 to 26.5) | 40.8 (40.3 to 41.3) | 1.0 (1.0 to 1.0) | 0.0 (0.0 to 0.0) | 87.2 (86.8 to 87.6) | 71.2 (70.2 to 72.1) | n/a | 0.00 |
| GPR only (dual band) | 83.6 (83.3 to 83.9) | 13.2 (13.0 to 13.5) | 1.1 (1.1 to 1.1) | 0.0 (0.0 to 0.0) | 59.4 (59.0 to 59.7) | 15.7 (15.4 to 16.0) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 84.0 (83.7 to 84.3) | 12.9 (12.7 to 13.2) | 1.1 (1.1 to 1.1) | 0.0 (0.0 to 0.0) | 59.2 (58.8 to 59.6) | 15.3 (15.0 to 15.6) | 0.016 | 0.01 |
| v1.0 continuity filter, one pass | 97.2 (97.0 to 97.4) | 2.5 (2.3 to 2.7) | 0.4 (0.3 to 0.4) | 16.0 (16.0 to 16.0) | 44.8 (44.4 to 45.1) | 2.2 (2.1 to 2.4) | 0.034 | 0.02 |
| v1.0 filter, three repeat passes | 98.1 (98.0 to 98.3) | 1.7 (1.5 to 1.9) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 44.5 (44.2 to 45.0) | 1.5 (1.4 to 1.6) | 0.034 | 0.03 |
| Proposed, one pass | 98.2 (98.1 to 98.4) | 1.6 (1.5 to 1.8) | 1.9 (1.8 to 2.0) | 16.3 (16.0 to 16.5) | 43.6 (43.1 to 44.0) | 1.4 (1.3 to 1.6) | 0.028 | 0.85 |
| Proposed, three parallel passes | 99.1 (99.0 to 99.2) | 0.8 (0.7 to 0.9) | 2.0 (1.9 to 2.1) | 15.9 (15.7 to 16.2) | 43.2 (42.7 to 43.6) | 0.7 (0.6 to 0.8) | 0.023 | 1.88 |

Zone shares (A/B/C): 0.72 / 0.09 / 0.20.

### B gpr failed flagged  (gpr_state=failed, flag=all; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.7 (25.0 to 26.3) | 59.2 (58.5 to 59.8) | 1.0 (1.0 to 1.0) | 0.0 (0.0 to 0.0) | 82.7 (82.2 to 83.2) | 97.9 (96.7 to 99.1) | n/a | 0.00 |
| GPR only (dual band) | 1.1 (1.1 to 1.2) | 65.9 (65.4 to 66.4) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 98.9 (98.8 to 98.9) | 130.2 (129.2 to 131.3) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 2.3 (2.2 to 2.4) | 65.6 (65.1 to 66.1) | 1.1 (1.1 to 1.1) | 0.0 (0.0 to 0.0) | 98.1 (98.1 to 98.2) | 128.8 (127.8 to 129.7) | 0.567 | 0.01 |
| v1.0 continuity filter, one pass | 2.0 (1.9 to 2.1) | 65.2 (64.7 to 65.9) | 0.0 (0.0 to 0.1) | 16.0 (16.0 to 16.0) | 82.9 (82.9 to 82.9) | 108.1 (107.2 to 109.2) | 0.572 | 0.02 |
| v1.0 filter, three repeat passes | 0.5 (0.4 to 0.5) | 65.6 (64.9 to 66.2) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 83.7 (83.7 to 83.8) | 109.8 (108.7 to 110.9) | 0.598 | 0.04 |
| Proposed, one pass | 100.0 (100.0 to 100.0) | 0.0 (0.0 to 0.0) | 25.8 (24.4 to 27.3) | 68.8 (68.5 to 69.1) | 6.5 (6.2 to 6.7) | 0.0 (0.0 to 0.0) | 0.100 | 0.79 |
| Proposed, three parallel passes | 100.0 (100.0 to 100.0) | 0.1 (0.1 to 0.1) | 25.7 (24.0 to 27.1) | 68.7 (68.4 to 68.9) | 6.5 (6.2 to 6.8) | 0.0 (0.0 to 0.0) | 0.088 | 1.69 |

Zone shares (A/B/C): 0.00 / 0.42 / 0.58.

### C gpr failed unflagged  (gpr_state=failed, flag=none; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.2 (24.6 to 26.0) | 58.6 (58.0 to 59.2) | 1.0 (1.0 to 1.0) | 0.0 (0.0 to 0.0) | 83.2 (82.7 to 83.7) | 97.5 (96.3 to 98.5) | n/a | 0.00 |
| GPR only (dual band) | 1.1 (1.1 to 1.2) | 65.2 (64.7 to 65.7) | 1.2 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 98.9 (98.8 to 98.9) | 128.9 (127.9 to 130.0) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 2.2 (2.1 to 2.3) | 65.0 (64.4 to 65.5) | 1.1 (1.1 to 1.1) | 0.0 (0.0 to 0.0) | 98.2 (98.1 to 98.2) | 127.5 (126.5 to 128.6) | 0.561 | 0.01 |
| v1.0 continuity filter, one pass | 2.0 (2.0 to 2.1) | 64.9 (64.4 to 65.6) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 82.9 (82.8 to 82.9) | 107.6 (106.7 to 108.8) | 0.567 | 0.02 |
| v1.0 filter, three repeat passes | 0.5 (0.4 to 0.6) | 65.3 (64.7 to 65.9) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 83.7 (83.7 to 83.8) | 109.3 (108.4 to 110.3) | 0.593 | 0.04 |
| Proposed, one pass | 1.9 (1.8 to 2.0) | 65.0 (64.3 to 65.6) | 0.6 (0.5 to 0.7) | 15.1 (15.0 to 15.3) | 83.7 (83.5 to 83.8) | 108.7 (107.6 to 109.8) | 0.576 | 0.79 |
| Proposed, three parallel passes | 3.4 (3.2 to 3.6) | 64.5 (63.9 to 65.1) | 0.6 (0.5 to 0.7) | 15.3 (15.2 to 15.5) | 82.6 (82.4 to 82.7) | 106.6 (105.5 to 107.5) | 0.567 | 1.70 |

Zone shares (A/B/C): 0.73 / 0.08 / 0.19.

### D both degraded  (gpr_state=degraded, em_state=degraded; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.9 (25.2 to 26.5) | 59.3 (58.7 to 60.0) | 1.0 (1.0 to 1.0) | 0.0 (0.0 to 0.0) | 82.6 (82.1 to 83.0) | 98.0 (96.9 to 99.3) | n/a | 0.00 |
| GPR only (dual band) | 78.2 (77.9 to 78.6) | 30.0 (29.4 to 30.6) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 47.9 (47.4 to 48.4) | 28.8 (28.2 to 29.3) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 79.1 (78.7 to 79.5) | 29.2 (28.6 to 29.7) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 47.4 (46.9 to 47.8) | 27.7 (27.1 to 28.1) | 0.067 | 0.01 |
| v1.0 continuity filter, one pass | 92.8 (92.5 to 93.2) | 12.2 (11.6 to 12.8) | 0.2 (0.2 to 0.2) | 16.0 (16.0 to 16.0) | 32.5 (31.9 to 33.0) | 7.9 (7.6 to 8.3) | 0.060 | 0.02 |
| v1.0 filter, three repeat passes | 93.3 (93.0 to 93.7) | 11.4 (10.8 to 12.1) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 32.2 (31.7 to 32.7) | 7.4 (7.0 to 7.8) | 0.072 | 0.03 |
| Proposed, one pass | 96.2 (95.9 to 96.5) | 7.0 (6.5 to 7.6) | 1.9 (1.7 to 2.1) | 19.7 (19.4 to 20.1) | 28.8 (28.3 to 29.3) | 4.1 (3.7 to 4.4) | 0.067 | 0.80 |
| Proposed, three parallel passes | 97.7 (97.5 to 97.9) | 4.3 (3.9 to 4.7) | 2.0 (1.8 to 2.2) | 19.3 (19.0 to 19.6) | 28.0 (27.4 to 28.5) | 2.4 (2.2 to 2.6) | 0.047 | 1.72 |

Zone shares (A/B/C): 0.67 / 0.11 / 0.23.

### mixed health realistic diagnostic  (gpr_state=mixed, em_state=mixed; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 22.4 (21.7 to 23.1) | 59.9 (59.4 to 60.6) | 1.0 (1.0 to 1.0) | 0.0 (0.0 to 0.0) | 84.9 (84.5 to 85.5) | 101.8 (100.6 to 103.1) | n/a | 0.00 |
| GPR only (dual band) | 75.2 (74.6 to 75.7) | 32.4 (31.7 to 33.1) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 50.3 (49.7 to 50.9) | 32.6 (31.9 to 33.4) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 75.7 (75.1 to 76.3) | 31.9 (31.3 to 32.6) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 50.0 (49.4 to 50.6) | 31.9 (31.1 to 32.6) | 0.091 | 0.01 |
| v1.0 continuity filter, one pass | 87.8 (87.2 to 88.4) | 18.9 (18.1 to 19.8) | 0.2 (0.2 to 0.2) | 16.0 (16.0 to 16.0) | 35.5 (34.9 to 36.1) | 13.5 (12.8 to 14.2) | 0.085 | 0.02 |
| v1.0 filter, three repeat passes | 88.1 (87.4 to 88.8) | 18.5 (17.5 to 19.4) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 35.4 (34.9 to 36.0) | 13.1 (12.4 to 13.8) | 0.097 | 0.03 |
| Proposed, one pass | 97.2 (96.9 to 97.5) | 5.3 (4.8 to 5.8) | 2.4 (2.2 to 2.6) | 21.7 (21.3 to 22.1) | 27.4 (26.9 to 27.9) | 2.9 (2.6 to 3.2) | 0.057 | 0.80 |
| Proposed, three parallel passes | 98.2 (97.9 to 98.4) | 3.5 (3.1 to 3.9) | 2.5 (2.2 to 2.7) | 21.4 (21.0 to 21.7) | 26.9 (26.4 to 27.4) | 1.9 (1.7 to 2.1) | 0.040 | 1.71 |

Zone shares (A/B/C): 0.64 / 0.12 / 0.24.

### map offset  (map_quality=offset, truth_mode=swath; 17500 corridors; truth label: swath)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 19.9 (19.3 to 20.6) | 61.1 (60.5 to 61.7) | 1.0 (1.0 to 1.0) | 0.0 (0.0 to 0.0) | 86.5 (86.1 to 86.9) | 105.7 (104.5 to 106.9) | n/a | 0.00 |
| GPR only (dual band) | 20.0 (19.4 to 20.6) | 61.1 (60.4 to 61.8) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 86.4 (86.0 to 86.8) | 105.6 (104.4 to 106.9) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 21.6 (21.0 to 22.2) | 60.6 (60.0 to 61.3) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 85.4 (85.0 to 85.8) | 103.5 (102.3 to 104.7) | 0.441 | 0.02 |
| v1.0 continuity filter, one pass | 24.7 (23.9 to 25.5) | 59.5 (58.8 to 60.3) | 0.1 (0.1 to 0.1) | 16.0 (16.0 to 16.0) | 70.3 (69.8 to 70.7) | 83.6 (82.4 to 84.9) | 0.442 | 0.03 |
| v1.0 filter, three repeat passes | 22.9 (22.1 to 23.7) | 60.0 (59.4 to 60.7) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 71.3 (70.8 to 71.8) | 85.6 (84.6 to 86.7) | 0.469 | 0.04 |
| Proposed, one pass | 99.9 (99.8 to 99.9) | 2.2 (1.2 to 3.2) | 73.9 (72.4 to 75.3) | 48.0 (47.6 to 48.5) | 2.2 (2.1 to 2.4) | 0.1 (0.1 to 0.1) | 0.052 | 0.82 |
| Proposed, three parallel passes | 99.2 (99.1 to 99.3) | 1.6 (1.4 to 1.9) | 3.5 (3.3 to 3.8) | 17.7 (17.5 to 17.9) | 26.2 (25.8 to 26.8) | 0.9 (0.7 to 1.0) | 0.013 | 1.79 |

Zone shares (A/B/C): 0.71 / 0.09 / 0.20.

### map incomplete  (map_quality=incomplete; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 26.0 (25.3 to 26.8) | 58.8 (58.1 to 59.3) | 1.0 (1.0 to 1.1) | 0.0 (0.0 to 0.0) | 82.6 (82.1 to 83.0) | 97.0 (95.7 to 98.1) | n/a | 0.00 |
| GPR only (dual band) | 83.4 (83.2 to 83.7) | 24.2 (23.7 to 24.7) | 1.2 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 44.9 (44.4 to 45.3) | 21.7 (21.3 to 22.1) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 84.0 (83.7 to 84.3) | 23.6 (23.1 to 24.1) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 44.5 (44.0 to 45.1) | 21.0 (20.6 to 21.3) | 0.038 | 0.01 |
| v1.0 continuity filter, one pass | 97.4 (97.2 to 97.6) | 4.8 (4.4 to 5.2) | 0.3 (0.2 to 0.3) | 16.0 (16.0 to 16.0) | 30.3 (29.8 to 30.8) | 2.9 (2.7 to 3.1) | 0.062 | 0.02 |
| v1.0 filter, three repeat passes | 98.0 (97.8 to 98.2) | 3.7 (3.3 to 4.1) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 30.0 (29.5 to 30.7) | 2.2 (2.0 to 2.5) | 0.066 | 0.04 |
| Proposed, one pass | 98.2 (98.1 to 98.4) | 3.3 (2.9 to 3.6) | 1.1 (1.0 to 1.2) | 17.0 (16.7 to 17.3) | 29.3 (28.6 to 29.7) | 1.9 (1.7 to 2.1) | 0.130 | 0.83 |
| Proposed, three parallel passes | 99.2 (99.1 to 99.3) | 1.5 (1.3 to 1.7) | 1.2 (1.1 to 1.3) | 16.6 (16.4 to 16.9) | 28.7 (28.2 to 29.2) | 0.9 (0.8 to 1.0) | 0.080 | 1.71 |

Zone shares (A/B/C): 0.71 / 0.09 / 0.20.

### map misleading  (map_quality=misleading; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.1 (24.1 to 26.0) | 31.0 (30.5 to 31.7) | 1.0 (1.0 to 1.0) | 0.0 (0.0 to 0.0) | 90.0 (89.6 to 90.4) | 55.9 (54.8 to 57.1) | n/a | 0.00 |
| GPR only (dual band) | 83.7 (83.1 to 84.2) | 8.9 (8.6 to 9.2) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 68.0 (67.5 to 68.7) | 12.1 (11.8 to 12.5) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 84.2 (83.7 to 84.7) | 8.7 (8.4 to 9.0) | 1.1 (1.1 to 1.1) | 0.0 (0.0 to 0.0) | 67.9 (67.3 to 68.5) | 11.8 (11.4 to 12.2) | 0.028 | 0.01 |
| v1.0 continuity filter, one pass | 97.5 (97.2 to 97.7) | 1.5 (1.3 to 1.7) | 1.0 (1.0 to 1.0) | 16.0 (16.0 to 16.0) | 53.0 (52.3 to 53.6) | 1.6 (1.4 to 1.8) | 0.045 | 0.02 |
| v1.0 filter, three repeat passes | 98.1 (97.8 to 98.4) | 1.1 (1.0 to 1.3) | 0.1 (0.1 to 0.1) | 16.0 (16.0 to 16.0) | 53.2 (52.5 to 53.8) | 1.2 (1.0 to 1.3) | 0.039 | 0.03 |
| Proposed, one pass | 98.4 (98.1 to 98.6) | 1.0 (0.9 to 1.1) | 1.9 (1.8 to 2.1) | 16.2 (15.9 to 16.4) | 52.0 (51.4 to 52.7) | 1.0 (0.9 to 1.2) | 0.031 | 0.80 |
| Proposed, three parallel passes | 99.2 (99.1 to 99.4) | 0.5 (0.4 to 0.5) | 2.0 (1.9 to 2.2) | 16.2 (15.9 to 16.5) | 51.5 (50.9 to 52.1) | 0.5 (0.4 to 0.6) | 0.035 | 1.76 |

Zone shares (A/B/C): 0.72 / 0.09 / 0.20.

### sparse coverage  (dropout=0.5; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 12.6 (12.2 to 12.9) | 62.0 (61.4 to 62.6) | 0.5 (0.5 to 0.5) | 0.0 (0.0 to 0.0) | 91.7 (91.4 to 92.0) | 113.6 (112.6 to 114.8) | n/a | 0.00 |
| GPR only (dual band) | 41.8 (41.6 to 42.0) | 52.1 (51.4 to 52.6) | 0.6 (0.5 to 0.6) | 0.0 (0.0 to 0.0) | 72.7 (72.4 to 73.0) | 75.6 (74.9 to 76.3) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 42.0 (41.8 to 42.2) | 52.0 (51.4 to 52.5) | 0.6 (0.5 to 0.6) | 0.0 (0.0 to 0.0) | 72.5 (72.3 to 72.8) | 75.3 (74.7 to 76.0) | 0.304 | 0.01 |
| v1.0 continuity filter, one pass | 97.4 (97.1 to 97.6) | 4.7 (4.3 to 5.1) | 2.2 (2.1 to 2.3) | 16.0 (16.0 to 16.0) | 30.4 (29.9 to 30.9) | 2.9 (2.6 to 3.1) | 0.024 | 0.03 |
| v1.0 filter, three repeat passes | 97.7 (97.5 to 97.9) | 4.0 (3.7 to 4.4) | 0.4 (0.4 to 0.4) | 16.0 (16.0 to 16.0) | 30.7 (30.2 to 31.3) | 2.5 (2.2 to 2.7) | 0.032 | 0.04 |
| Proposed, one pass | 98.5 (98.4 to 98.7) | 2.7 (2.5 to 3.1) | 3.6 (3.5 to 3.8) | 16.5 (16.3 to 16.7) | 28.9 (28.4 to 29.4) | 1.6 (1.4 to 1.8) | 0.047 | 0.80 |
| Proposed, three parallel passes | 99.2 (99.1 to 99.3) | 1.6 (1.4 to 1.8) | 3.7 (3.6 to 3.9) | 16.2 (16.0 to 16.5) | 28.5 (28.0 to 29.0) | 0.9 (0.8 to 1.0) | 0.030 | 1.69 |

Zone shares (A/B/C): 0.72 / 0.09 / 0.20.

### ood soils  (soils=[5, 6], guard=True; 7000 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.7 (24.7 to 26.9) | 57.5 (56.3 to 58.5) | 1.0 (1.0 to 1.1) | 0.0 (0.0 to 0.0) | 83.1 (82.4 to 83.9) | 95.5 (93.3 to 97.4) | n/a | 0.00 |
| GPR only (dual band) | 52.5 (51.7 to 53.3) | 47.0 (45.9 to 48.1) | 3.4 (3.3 to 3.5) | 0.0 (0.0 to 0.0) | 65.1 (64.3 to 65.7) | 61.1 (59.8 to 62.4) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 55.1 (54.2 to 56.0) | 45.5 (44.5 to 46.4) | 3.1 (3.0 to 3.2) | 0.0 (0.0 to 0.0) | 63.5 (62.8 to 64.2) | 57.8 (56.6 to 59.0) | 0.189 | 0.01 |
| v1.0 continuity filter, one pass | 67.7 (66.8 to 68.6) | 37.0 (35.6 to 38.3) | 0.3 (0.3 to 0.4) | 20.0 (20.0 to 20.0) | 45.0 (44.3 to 45.7) | 33.3 (32.2 to 34.5) | 0.175 | 0.01 |
| v1.0 filter, three repeat passes | 68.0 (66.9 to 69.1) | 36.7 (35.4 to 38.0) | 0.0 (0.0 to 0.0) | 20.0 (20.0 to 20.0) | 45.0 (44.1 to 45.7) | 33.0 (31.7 to 34.2) | 0.194 | 0.02 |
| Proposed, one pass | 82.6 (81.2 to 84.1) | 22.7 (21.2 to 24.5) | 2.1 (1.7 to 2.6) | 58.6 (58.0 to 59.2) | 19.8 (19.0 to 20.4) | 9.0 (8.2 to 9.8) | 0.237 | 0.27 |
| Proposed, three parallel passes | 94.5 (93.8 to 95.3) | 8.7 (7.7 to 9.9) | 2.1 (1.7 to 2.6) | 57.7 (57.0 to 58.4) | 16.7 (16.0 to 17.4) | 2.9 (2.5 to 3.3) | 0.166 | 0.60 |

Zone shares (A/B/C): 0.64 / 0.10 / 0.27. Reference check flagged 35.3% of corridors.

### ood soils no guard  (soils=[5, 6], guard=False; 7000 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 24.1 (22.9 to 25.3) | 59.1 (58.2 to 60.0) | 1.0 (1.0 to 1.1) | 0.0 (0.0 to 0.0) | 83.9 (83.0 to 84.7) | 99.1 (97.4 to 101.1) | n/a | 0.00 |
| GPR only (dual band) | 52.3 (51.6 to 53.2) | 48.2 (47.2 to 49.2) | 3.4 (3.3 to 3.5) | 0.0 (0.0 to 0.0) | 64.7 (63.9 to 65.3) | 62.3 (60.9 to 63.6) | n/a | 0.00 |
| Per-segment GPR + EM fusion | 54.6 (53.8 to 55.4) | 46.9 (45.9 to 47.8) | 3.1 (3.0 to 3.2) | 0.0 (0.0 to 0.0) | 63.2 (62.5 to 64.1) | 59.3 (58.1 to 60.4) | 0.197 | 0.01 |
| v1.0 continuity filter, one pass | 66.7 (65.6 to 67.7) | 38.8 (37.5 to 40.1) | 0.3 (0.3 to 0.4) | 20.0 (20.0 to 20.0) | 45.0 (44.2 to 45.6) | 34.9 (33.7 to 36.2) | 0.185 | 0.01 |
| v1.0 filter, three repeat passes | 66.7 (65.5 to 67.8) | 38.7 (37.5 to 40.0) | 0.0 (0.0 to 0.0) | 20.0 (20.0 to 20.0) | 45.0 (44.3 to 45.9) | 34.9 (33.7 to 36.3) | 0.205 | 0.02 |
| Proposed, one pass | 62.5 (61.2 to 63.8) | 40.8 (39.3 to 42.2) | 1.7 (1.4 to 2.0) | 24.5 (24.1 to 24.9) | 44.7 (43.8 to 45.4) | 36.5 (34.8 to 37.8) | 0.245 | 0.28 |
| Proposed, three parallel passes | 75.2 (74.1 to 76.1) | 31.9 (30.7 to 33.5) | 1.7 (1.4 to 1.9) | 23.4 (23.1 to 23.9) | 38.7 (37.9 to 39.5) | 24.7 (23.7 to 25.9) | 0.175 | 0.60 |

Zone shares (A/B/C): 0.63 / 0.10 / 0.26.

### combined stress  (geom=shifted, gpr_state=mixed, em_state=mixed, map_quality=offset, dropout=0.3, truth_mode=swath; 17500 corridors; truth label: swath)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 9.7 (9.3 to 10.0) | 63.5 (62.9 to 64.1) | 0.7 (0.7 to 0.7) | 0.0 (0.0 to 0.0) | 93.4 (93.2 to 93.6) | 118.6 (117.6 to 119.8) | n/a | 0.00 |
| GPR only (dual band) | 15.2 (14.8 to 15.6) | 62.0 (61.4 to 62.6) | 0.8 (0.8 to 0.8) | 0.0 (0.0 to 0.0) | 89.8 (89.5 to 90.0) | 111.4 (110.1 to 112.4) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 15.8 (15.4 to 16.1) | 61.9 (61.3 to 62.4) | 0.8 (0.8 to 0.8) | 0.0 (0.0 to 0.0) | 89.4 (89.1 to 89.6) | 110.6 (109.6 to 111.4) | 0.476 | 0.01 |
| v1.0 continuity filter, one pass | 26.9 (26.3 to 27.5) | 58.4 (57.7 to 59.1) | 0.3 (0.3 to 0.3) | 16.0 (16.0 to 16.0) | 69.1 (68.7 to 69.4) | 80.6 (79.7 to 81.7) | 0.423 | 0.02 |
| v1.0 filter, three repeat passes | 25.2 (24.6 to 25.9) | 58.8 (58.1 to 59.5) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 70.1 (69.7 to 70.5) | 82.5 (81.4 to 83.6) | 0.448 | 0.03 |
| Proposed, one pass | 97.3 (97.1 to 97.6) | 28.0 (25.7 to 30.5) | 65.5 (63.6 to 67.5) | 59.8 (59.3 to 60.3) | 3.2 (3.0 to 3.4) | 1.8 (1.6 to 2.0) | 0.119 | 0.80 |
| Proposed, three parallel passes | 93.0 (92.6 to 93.4) | 13.4 (12.6 to 14.0) | 6.7 (6.3 to 7.2) | 24.8 (24.4 to 25.2) | 26.5 (26.0 to 27.0) | 7.1 (6.7 to 7.5) | 0.052 | 1.74 |

Zone shares (A/B/C): 0.64 / 0.12 / 0.24.

### baseline with reference check  (guard=True; 17500 corridors; truth label: footprint)

| Model | Sensitivity | False clearance | False alarm | Unverified | Released | Strikes/km | ECE | Compute (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM only | 25.4 (24.7 to 26.2) | 59.0 (58.4 to 59.7) | 1.0 (1.0 to 1.0) | 0.0 (0.0 to 0.0) | 82.9 (82.4 to 83.4) | 97.9 (96.6 to 99.2) | n/a | 0.00 |
| GPR only (dual band) | 83.9 (83.6 to 84.2) | 23.7 (23.2 to 24.2) | 1.1 (1.1 to 1.2) | 0.0 (0.0 to 0.0) | 44.5 (44.0 to 45.0) | 21.1 (20.7 to 21.5) | n/a | 0.01 |
| Per-segment GPR + EM fusion | 84.3 (84.1 to 84.6) | 23.2 (22.7 to 23.8) | 1.1 (1.1 to 1.1) | 0.0 (0.0 to 0.0) | 44.2 (43.7 to 44.7) | 20.6 (20.2 to 20.9) | 0.036 | 0.01 |
| v1.0 continuity filter, one pass | 97.7 (97.5 to 97.9) | 4.2 (3.8 to 4.6) | 0.3 (0.2 to 0.3) | 16.0 (16.0 to 16.0) | 30.1 (29.6 to 30.6) | 2.5 (2.3 to 2.7) | 0.029 | 0.02 |
| v1.0 filter, three repeat passes | 98.3 (98.0 to 98.4) | 3.2 (2.8 to 3.6) | 0.0 (0.0 to 0.0) | 16.0 (16.0 to 16.0) | 29.8 (29.3 to 30.3) | 1.9 (1.7 to 2.1) | 0.039 | 0.03 |
| Proposed, one pass | 98.5 (98.4 to 98.7) | 2.8 (2.5 to 3.1) | 1.6 (1.5 to 1.7) | 15.9 (15.7 to 16.2) | 29.1 (28.5 to 29.6) | 1.6 (1.4 to 1.8) | 0.044 | 0.82 |
| Proposed, three parallel passes | 99.3 (99.2 to 99.4) | 1.3 (1.1 to 1.5) | 1.7 (1.5 to 1.8) | 15.6 (15.4 to 15.9) | 28.7 (28.2 to 29.2) | 0.7 (0.6 to 0.8) | 0.027 | 1.73 |

Zone shares (A/B/C): 0.72 / 0.09 / 0.19. Reference check flagged 0.2% of corridors.

# S2. Equal-coverage clearance (all policies, both worlds)

### Matched model

| Policy | 5% | 10% | 20% | 30% | 40% | 50% | 60% | 70% | max release | cov. at <=1% | <=2% | <=5% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM locator only | 5.9 | 11.9 | 23.8 | 35.6 | 47.5 | 59.4 | 71.2 | 83.1 | 100% | 0.0% | 0.0% | 0.0% |
| GPR dual-band, per segment | 0.0 | 0.0 | 0.1 | 3.3 | 15.5 | 31.8 | 51.7 | 71.7 | 100% | 24.5% | 26.1% | 29.7% |
| GPR + EM fusion, per segment | 0.0 | 0.0 | 0.1 | 3.1 | 15.3 | 31.8 | 51.7 | 71.7 | 100% | 24.7% | 26.4% | 30.0% |
| Anchored filter, binary map | 0.0 | 0.0 | 0.0 | 2.1 | 13.4 | 31.7 | 51.7 | 71.7 | 100% | 27.1% | 28.9% | 31.0% |
| Anchored filter, three-state (unverified never released) | 0.0 | 0.0 | 0.0 | 2.8 | 22.6 | 42.6 | 62.6 | 82.6 | 84% | 27.1% | 28.9% | 30.2% |
| Random release (reference) | 6.6 | 13.2 | 26.3 | 39.5 | 52.7 | 65.8 | 79.0 | 92.2 | 100% | 0.0% | 0.0% | 0.0% |

### Half of utility losses are lateral shifts

| Policy | 5% | 10% | 20% | 30% | 40% | 50% | 60% | 70% | max release | cov. at <=1% | <=2% | <=5% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EM locator only | 6.2 | 12.5 | 24.9 | 37.4 | 49.9 | 62.4 | 74.9 | 87.4 | 100% | 0.0% | 0.0% | 0.0% |
| GPR dual-band, per segment | 0.0 | 0.1 | 6.9 | 19.7 | 35.4 | 51.4 | 67.8 | 83.8 | 100% | 12.1% | 12.7% | 14.5% |
| GPR + EM fusion, per segment | 0.0 | 0.1 | 6.8 | 19.7 | 35.2 | 51.7 | 67.7 | 83.8 | 100% | 12.1% | 12.8% | 14.5% |
| Anchored filter, binary map | 0.0 | 0.0 | 5.7 | 18.7 | 34.8 | 51.4 | 67.4 | 83.5 | 100% | 13.1% | 14.3% | 15.8% |
| Anchored filter, three-state (unverified never released) | 0.0 | 0.0 | 7.7 | 23.7 | 39.9 | 56.1 | 72.3 | 88.2 | 84% | 13.1% | 14.3% | 15.7% |
| Random release (reference) | 6.6 | 13.2 | 26.4 | 39.6 | 52.8 | 66.0 | 79.2 | 92.3 | 100% | 0.0% | 0.0% | 0.0% |

Strikes per km of corridor. Policies rank segments by probability (EM only and GPR only by detection); the three-state rule never releases unverified segments, so its maximum release is limited.

# S3. Capability zones: deepest depth (m) of zone A and zone B

| Soil | pe water | ci water | plastic sewer | concrete sewer | power | telephone | fibre |
| --- | --- | --- | --- | --- | --- | --- | --- |
| dry sand | 4.5 / 6.0 | 7.0 / 7.0 | 5.0 / 7.0 | 5.5 / 7.0 | 6.5 / 7.0 | 5.0 / 7.0 | 4.0 / 6.0 |
| dry loam | 3.0 / 4.0 | 4.5 / 7.0 | 3.5 / 4.5 | 3.5 / 4.5 | 4.0 / 7.0 | 3.0 / 6.5 | 2.5 / 4.0 |
| moist loam | 2.0 / 2.5 | 3.0 / 6.0 | 2.0 / 3.0 | 2.5 / 3.0 | 2.5 / 6.0 | 2.0 / 5.0 | 2.0 / 2.5 |
| clay | 1.5 / 2.0 | 2.0 / 4.5 | 1.5 / 2.0 | 1.5 / 2.0 | 2.0 / 4.5 | 1.5 / 3.5 | 1.5 / 2.0 |
| rubble fill | 2.0 / 3.5 | 3.5 / 7.0 | 2.5 / 3.5 | 2.5 / 4.0 | 3.0 / 6.5 | 2.5 / 5.5 | 2.0 / 3.0 |
| wet clay | 0.5 / 1.0 | 1.0 / 3.5 | 1.0 / 1.0 | 1.0 / 1.0 | 1.0 / 3.5 | 1.0 / 3.0 | 0.5 / 1.0 |
| saline fill | 1.5 / 2.0 | 2.0 / 5.0 | 1.5 / 2.5 | 1.5 / 2.5 | 2.0 / 5.0 | 1.5 / 4.0 | 1.5 / 2.0 |

Entries are A / B: the deepest depth with fused signal detection probability at least 0.9 (zone A) and at least 0.3 (zone A or B), on a 0.5 m grid up to 7 m. Wet clay and saline fill are the unseen soils.

| Soil | Two-way attenuation at 250 MHz (dB/m) | Assumed relative permittivity | Implied conductivity (mS/m) |
| --- | --- | --- | --- |
| dry sand | 1.5 | 4 | 0.9 |
| dry loam | 4.0 | 8 | 3.5 |
| moist loam | 8.0 | 15 | 9.5 |
| clay | 14.0 | 25 | 21.4 |
| rubble fill | 6.0 | 8 | 5.2 |
| wet clay | 30.0 | 25 | 45.8 |
| saline fill | 12.0 | 12 | 12.7 |

Implied conductivity from the one-way low-loss relation alpha = 1636 sigma / sqrt(eps_r) dB/m. The permittivities are assumptions; the comparison with site measurements is part of any field calibration.

# S4. Value of information: change in total cost against no pits (US$ thousand per km)

### Holding cost US$1,000 per segment

| Strategy | 0% | 1% | 2% | 5% | 10% | 20% | 40% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| random | 0.0 (+-0.0) | 0.5 (+-0.1) | 1.0 (+-0.2) | 2.6 (+-0.5) | 5.2 (+-0.9) | 10.3 (+-1.8) | 20.5 (+-3.5) |
| probability | 0.0 (+-0.0) | 0.9 (+-0.0) | 1.7 (+-0.0) | 4.3 (+-0.0) | 8.6 (+-0.1) | 16.8 (+-0.5) | 32.5 (+-1.7) |
| value of information | 0.0 (+-0.0) | -0.1 (+-0.1) | -0.1 (+-0.1) | -0.1 (+-0.1) | -0.1 (+-0.1) | -0.1 (+-0.1) | -0.1 (+-0.1) |

### Holding cost US$3,000 per segment

| Strategy | 0% | 1% | 2% | 5% | 10% | 20% | 40% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| random | 0.0 (+-0.0) | 0.4 (+-0.2) | 0.7 (+-0.3) | 1.9 (+-0.7) | 3.7 (+-1.3) | 7.5 (+-2.6) | 14.8 (+-5.2) |
| probability | 0.0 (+-0.0) | 0.9 (+-0.0) | 1.7 (+-0.0) | 4.3 (+-0.0) | 8.4 (+-0.2) | 15.7 (+-1.3) | 28.3 (+-4.9) |
| value of information | 0.0 (+-0.0) | -0.5 (+-0.6) | -0.6 (+-0.7) | -0.9 (+-1.2) | -1.2 (+-1.7) | -1.5 (+-2.4) | -1.5 (+-2.4) |

### Holding cost US$10,000 per segment

| Strategy | 0% | 1% | 2% | 5% | 10% | 20% | 40% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| random | 0.0 (+-0.0) | 0.3 (+-0.2) | 0.5 (+-0.3) | 1.4 (+-0.7) | 2.8 (+-1.5) | 5.5 (+-3.0) | 10.8 (+-6.0) |
| probability | 0.0 (+-0.0) | 0.9 (+-0.0) | 1.7 (+-0.0) | 4.3 (+-0.1) | 8.0 (+-0.7) | 13.5 (+-4.0) | 21.4 (+-12.0) |
| value of information | 0.0 (+-0.0) | -1.0 (+-1.0) | -1.4 (+-1.4) | -2.2 (+-2.5) | -3.3 (+-3.8) | -4.9 (+-6.3) | -5.5 (+-7.3) |

# S5. Pilot sample sizes (exact binomial, two-sided 95% interval)

| Target upper bound for a failure rate | Observed failures | Segments required |
| --- | --- | --- |
| 1% | 0 | 368 |
| 1% | 1 | 555 |
| 1% | 2 | 720 |
| 1% | 5 | 1164 |
| 2% | 0 | 183 |
| 2% | 1 | 277 |
| 2% | 2 | 359 |
| 2% | 5 | 581 |
| 5% | 0 | 72 |
| 5% | 1 | 110 |
| 5% | 2 | 142 |
| 5% | 5 | 230 |

| Target lower bound for sensitivity | Observed misses | Occupied segments required |
| --- | --- | --- |
| 90% | 0 | 36 |
| 90% | 1 | 54 |
| 90% | 2 | 70 |
| 90% | 5 | 114 |
| 95% | 0 | 72 |
| 95% | 1 | 110 |
| 95% | 2 | 142 |
| 95% | 5 | 230 |